SMART CITY INFRASTRUCTURE

Submitted By

Muhammad Hassan Sohail 22K-8704

Group Members

Muhammad Hassan Sohail 22K-8704

Ahmed Arshad 22K-8725

Department of Computer Science
National University of Computer & Emerging Sciences

- Motivation

With the rapid urbanization and increased population in metropolitan areas, the need for efficient, secure, and automated infrastructure has become imperative. This project aims to design and simulate a smart city infrastructure that integrates advanced networking concepts and IoT devices to enhance the quality of life, resource management, and security of urban dwellers.

- Overview

2.1 Significance of the Project

The project is significant due to its applicability in modern urban               development. It provides a practical implementation of various networking concepts such as IP addressing, VLAN, OSPF, subnetting, and DHCP, alongside IoT and wireless technologies. Academically, it offers a platform to understand real-world applications of computer networks.

2.2 Description of the Project

The Smart City Infrastructure project simulates a citywide network connecting smart homes, management offices, departments, and service providers. It includes DHCP, DNS, FTP, Email, IoT, VLAN, and routing protocols to manage network services efficiently. Different clusters such as Smart Homes, Management Offices, and City Departments are interlinked with redundancy to ensure constant communication and data flow.

2.3 Background of the Project

Numerous research and practical implementations have explored smart cities. References such as Cisco Smart+Connected Communities, IEEE publications on smart infrastructure, and simulation tools like Cisco Packet Tracer provide foundational knowledge. Concepts from computer networking courses and IoT system design were also pivotal.

2.4 Project Category

Product-based Project

- Features / Scope / Modules

· Independent Smart Homes
Configured with HomeGateway (SSID: HomeGateway, IP: 192.168.25.1), equipped with IoT devices controlled via set conditions.

· Management Office Integration
Interlinked with Parking, Energy, and ISP clusters. Provides DNS, DHCP, Email, HTTP/HTTPS, and IoT services.

· DHCP Centralized Configuration
Dynamic IP allocation from server site to various clusters (Parking, Energy, Departments).

· Static Configuration for Server Site
Manages DNS, FTP, Email, and DHCP services with IP range 192.168.102.64/28.

· VLAN and Inter-VLAN Routing
Implemented across switches and multilayer devices to ensure segmented and secure communication.

· OSPF and Default Routing
Enabled on routers and multilayer switches to ensure optimal path routing and redundancy.

· Wireless Communication via Cell Towers
All mobile devices connect to the nearest cell tower based on the ISP.

- Project Planning

- Phase 1: Research & Requirement Gathering (Week 1-2)

- Phase 2: Network Topology Design (Week 3-4)

- Phase 3: Configuration & Implementation (Week 5-8)

- Phase 4: Testing & Troubleshooting (Week 9-10)

- Phase 5: Documentation & Final Review (Week 11-12)

Responsibilities:

- Muhammad Hassan Sohail: Topology design, DHCP & IoT configuration

- Ahmed Arshad: VLAN setup, routing protocols, documentation

- Project Feasibility

· Technical Feasibility
The project is feasible with current simulation tools like Cisco Packet Tracer. IoT and networking technologies used are widely available.

· Economic Feasibility
Minimal hardware/software costs due to use of simulators. High cost-efficiency with significant academic value.

· Schedule Feasibility
Planned within 3-month academic timeline. Buffer included for testing and documentation.

- Hardware and Software Requirements

Hardware:

- Computers with standard configurations

- Internet access

Software:

- Cisco Packet Tracer

- Any supporting text editor for documentation

- Diagrammatic Representation of the Overall System

- References

[1] Cisco Systems, "Smart+Connected Communities," https://www.cisco.com/c/en/us/solutions/industries/smart-connected-communities.html

[2] Tanenbaum, A. S., & Wetherall, D. J. (2011). Computer Networks (5th ed.). Pearson.

[3] IEEE Smart Cities Initiative, https://smartcities.ieee.org/

[4] Packet Tracer Simulation Examples and Documentation

[5] IoT and Smart City Concepts, Research Articles on IEEE Xplore

Working Demo Screenshots: